[image: image1.png]TAMARAC
— ~

The City For Your Life



        CITY OF TAMARAC
PUBLIC SERVICES DEPARTMENT

                                                             Engineering Division

                   SHOP DRAWING REVIEW CHECKLIST



Project Name: _________________________________________    Date: ___/___/___

YES
NO

(
(
Each set of shop drawings has been signed as reviewed by the engineer             
of record.



___________________________________________________________

· Each page shall display the following information:

YES
NO

 (
(
Project name

 (
(
Structure number

 (
(
Structure type (such as FDOT “C” inlet)

 (
(
Top and side views of each structure

 (
(
Pipe opening locations shown on top view (or side view).



___________________________________________________________

· Each pipe opening shall display the following information:

YES
NO

 (
(
Distance from top of slab to bottom of pipe opening, or pipe invert



(Often shown as x inches up)

 (
(
Size of the opening (often shown as y inches Ø)



___________________________________________________________

· The following specifications shall be shown on each page:

YES
NO

 (
(
Compressive strength of concrete shall be no less than 4,000 psi.

 (
(
Wire mesh yield strength shall be no less than 65 ksi.

 (
(
Rebar yield strength shall be no less than 40 ksi (it can be 60 ksi).

 (
(
Structure walls and slabs must be no less than 8” thick

 (
(
Minimum sump depth shall be no less than 18” 



(from top of slab to invert of deepest pipe)



__________________________________________________________

· Locate the latest set of engineering plans for the project and check:

YES
NO

 (
(
Each structure number on the shop drawings shall coincide with the 

structure numbers shown on the plans.

 (
(
The number, location and direction of each pipe opening shown on the 

shop drawings shall coincide with the number and directions of the pipes shown on the plans.

 (
(
Each opening size shall be adequate to accommodate the pipe size 

shown on the engineering plans (opening must be larger than O.D.).

 (
(
Inverts and rim elevations shown on the drawings must coincide with 

those shown on the plans.



___________________________________________________________

· Check that the correct frames and grates are specified:

YES
NO

 (
(
For inlets, the frame and grate shall be USF#4155-6212.

 (
(
For manholes, the frame and lid shall be USF#420-B with the inscriptions 

“STORM SEWER”.

 (
(
Other structures will have to be reviewed on a case-by-case basis



including ditch inlets, curb inlets, inlets in retention areas, and gutter inlets.



___________________________________________________________

Additional Comments:

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

Revised 11/27/18
Page 2 of 2

